This document pertains to using RealTrak in conjunction with KCT511

by Joe Barger, N6KK.

KCT511's Tuner interface, named Tuner.COM, may be driven in one of

two ways:

1) Table Mode: The KCT Table is filled by RealTrak using either

the <Alt-F> command for one pass of one satellite, or by using

the Priority List function (Ctrl-PgDn) for multiple passes of

several satellites.

For Tabled AutoTuning information to be useful, the Beacon #1

frequency for each satellite MUST match the base frequency stored

in the KCT511 database (F3 & F4) exactly.  Otherwise, the doppler-

corrected frequency output by KCT511 will be in error.

2) Direct-Drive via OrbitDRV (by Paul Williamson, KB5MU).  In this mode,

the doppler information is fed to KCT511 directly, without reference

to the Table in DRV.COM.

To use Direct-Drive mode, the Table must be emptied (using the X

command in the <Alt-O> pop-up window.

OrbitDRV must be activated for the selected satellite, using the

<Crtl-R>, then <Ctrl-T> keys in RealTrak.  You may optionally quit

RealTrak by pressing <Ctrl-Q>.  Once OrbitDRV is activated, enter

the Tuner pop-up using whatever hot-key you have designated.  Then,

activate real-time tuning by pressing <M>.  If the KCT511 Tuner data-

base has been filled out correctly, autotuning should commence.

Remember that Direct-Drive autotuning is entirely under the control of

OrbitDRV.  The Beacon frequencies stored in RealTrak are irrelevant

here.

Do not try to mix these two modes.  For example, if you want to use the

Table Mode, be SURE to de-activate OrbitDRV's autotuning function.  Do

this by going to the TSR Status menu in RealTrak or by toggling <Ctrl-T>

until the little triangle disappears from between the beacon frequencies

in the RealTrak display.

M.R. Owen

4/18/93