����������������������������������������������������������

������������������������͸

�    C  L  U  S  S  E    �

������������������������;

The Free Finnish DX Cluster / Conference System

Version 0.30f - Still Under Construction !

����������������������������������������������������������

Written by Heikki Hannikainen, OH7LZB

Details about contacting the author in section 8.

����������������������������������������������������������

��� Index ���������������������������������������������������������������

0. Disclaimer and a licence to use

1. Introduction to Clusse

1.1 Design criteria

1.2 Features

1.3 Requirements

1.4 Terminology

1.5 About this file

1.6 Other free DX-cluster software available

2. Installation

2.1 First installation

2.2 Upgrading from a previous version

2.3 Some notes on configuring the BPQ switch

2.4 Some notes on using the PC/FlexNet kernel

3. Running Clusse

3.1 Command line parameters

3.2 Using the console

3.2.1 Keys

3.2.1 Console screen layout

3.2.3 Monitoring window layout

3.3 Other general (but important) stuff

3.4 Exit codes

3.5 Multitasking environments

3.6 Memory requirements

4. DX Cluster networking

4.1 Linking to the PacketCluster network

4.2 Network topology

4.3 Listen-only links

4.4 PacketCluster version compatibility

4.5 Linking two Clusse systems ( - not! )

5. Files

5.1 File descriptions and paths

5.2 Databases

5.2.1 PacketCluster-type databases

5.2.2 The DB0SDX QSL database

5.2.3 The DXCC database

5.2.4 The Buckmaster HamCall CD-ROM callbook

5.2.5 Other CD-ROM callbooks

5.2.6 Other databases available

6. Multicasting user interface

7. Troubleshooting

8. Getting the latest version & contacting the author

8.1 Clusse distribution sites

8.2 Clusse mailing lists

8.3 The author

Appendices:

A: Things To Do & Known (But Still Unresolved) Bugs

B: PC protocol message description

C: Clusse binary files description

D: Keeping the lawyers happy

For a QUICK introduction on the software take a look at intro.txt.

For a report on the development history, take a look at whatsnew.txt.

��� 0. Disclaimer and a licence to use ����������������������������������

CLUSSE IS PROVIDED "AS IS" WITHOUT WARRANTY OF ANY KIND. THE

ENTIRE RISK OF USING THIS PRODUCT IS ASSUMED BY YOU. IN NO

EVENT WILL THE AUTHOR BE LIABLE FOR ANY DAMAGES WHATSOEVER

ARISING OUT OF YOUR USE OR INABILITY TO USE THE SOFTWARE. BY

USING THIS PRODUCT, YOU AGREE TO THE ABOVE LIMITATIONS.

I hereby grant any licenced amateur radio operator to use this

software free of charge. You can modify the program as much as

you like, as long as you don't remove my signatures or distribute

the modified versions without my written permission. This software

may not be used in CB (Citizens Band, 27 MHz) or commercial

environment without my written permission.

Clusse is not for sale. You may not charge ANY money for

a copy of Clusse, except for the price of the media used

for transferring (the disk or the phone bill). Clusse may

be bundled on a CD-ROM or any other such commercial

package, for the price of sending me a free copy of the

product it is bundled with (1 copy per each different product).

If you think you could utilize Clusse in a commercial environment

and you are ready to pay for it, contact the author.

��� 1. Introduction to Clusse �������������������������������������������

1.1 Design criteria

For several years hams have used so called DX clusters for

distributing information over packet radio. DX spots, weather

warnings and other announcements. The whole network has been

using the PacketCluster (TM) commercial software package from

Pavillion Software.

I wanted to to put up a DX cluster node for the locals to

use, but found out that the only software available was a bit

too expensive for our club to buy. What the heck, i thought,

i didn't like it's user interface anyway...

When constructing Clusse, i've tried to avoid imitating the

PacketCluster. I forgot the original VMS-stylish user interface,

and made Clusse look more like any BBS or NOS mailbox. Just because

i personally don't like the VMS interface too much. I don't really

care if it's uncompatible, i'm NOT writing a CLONE! No complaints

on this one, please!

I intend to include all of the original PacketCluster features,

fix the things that are wrong in it and add a bunch of new

features. If there's a thing you'd like to see in Clusse, please

let me know!

1.2 Features

These are the most important features i've already included.

They're not in order of importance 8-)

- Most of the original PacketCluster features: DX, Announces, Talk,

Wx, WWV, Users/Nodes information, databases...

- Links with a PacketCluster (TM) notwork using the stupid PC protocol.

- A conversd-style conference mode with virtual channels.

- An indexed help system: The help file is available in Finnish,

English, Italian, Spanish, Swedish, Norwegian, Czech and German, and

you are invited to translate the file to your own language.

- Supports PacketCluster-type databases and the Buckmaster HamCall

CD-ROM.

- User-selectable character conversion tables - solves problems with

different systems to express special language-specific characters

(Scandinavia, Germany).

- Uses the G8BPQ Packet Switch TSR code to interface different kinds

of modem hardware. Thanks to John Wiseman for writing the switch!

This saves me from including support for many different types

of interfaces and including ax.25 & net/rom code in Clusse...

- The PC/FlexNet kernel can be used instead of the G8BPQ switch in

areas, where FlexNet is used for routing.

1.3  Requirements

- A 286 compatible computer with about 400 Kb of free base memory and

a hard disk (not required but VERY strongly suggested).

- MS-DOS 3.0 or newer (Windows 95 & Windows NT & OS/2 can all run

DOS programs like Clusse, Win95 is used for the developing work).

- G8BPQ packet switch version 4.05 or newer, or alternatively,

PC/FlexNet kernel (3.3e verified to work).

- The G8BPQ switch can currently (v4.08a) interface all KISS-type

TNC's, TheNet-type node stacks running the net/rom interlink protocol,

other systems that know either KISS or net/rom async (NOS, BPQ),

Ethernet for interfacing other BPQ and PE1CHL NET switches, many

brands of HDLC card modems (PacComm PC120, DRSI PC-PA, Thor RCL100,

BayCom USCC-Karte, PA0HZP OptoPcSCC).

- PC/FlexNet 3.3e has drivers for (at least) the following hardware:

* TNC2 TNC's with 6PACK firmware (a KISS driver is supplied, but

it is claimed to not work well with TNC's, 6PACK overcomes the

timing problems of KISS)

* BayCom modems (1200 bit/s serial port modem, 9600 bit/s paraller

port, USCC and SCC cards)

* Some sound cards and DSP systems

* Ethernet trough the following drivers:

IPX, Packet Drivers, AXIP (only inside a LAN), G8BPQ IPX

* Lots more... they're writing more drivers too fast for me to

keep up with!

1.4 Terminology

A DX Cluster is a group of DX Cluster nodes which talk to each

other. One Clusse or PacketCluster or clx copy running on a

computer makes up a DX cluster node. It's just a node of a

cluster, not a dx cluster. In short, a DX node. The terminology

and the cluster consept comes from the VAX/VMS world.

DX Cluster nodes are not the same as packet network nodes, like

TheNet/BPQ/CHL-Net/TexNet/FlexNet/you-name-it nodes. The G8BPQ

switch used when running Clusse can act as a net/rom network node.

The PC/FlexNet kit includes a FlexNet digipeater TSR.

PacketCluster (TM) is a DX-Cluster node software created by

Pavillion Software. Clusse is NOT a PacketCluster, it's just

another DX cluster node software package (a bit cheaper, though).

Clusse may NOT be called a PacketCluster, because PacketCluster

is a trademark of Pavillion Software. PacketCluster is often

referred to as "PC".

1.5 About this file

Like most of us programmers i like programming, but i'm not into

documenting. I'm sorry, but i can not do much about it. This

manual is not very accurate and does not go into all the details

of the software, but it's better than nothing: what can you expect

to get for free?

All of the user commands are documented in the user help files.

The configuration files have documentation in them, and this file

(clusse.doc) is not intended to document those files nor the user

commands. You can leard things by reading the whatsnew.txt and the

user help file through.

1.6 Other free DX-cluster software available

CLX runs on the excellent operating system Linux, and its user

interface is mostly compatible with the PacketCluster. So, if

you're already familiar with Linux, and you're after a

PacketCluster clone, go for clx! It's available on the Internet

on ftp.funet.fi, /pub/ham/unix/Linux/cluster/. The only drawbacks

are the hardware requirements of Linux (i'd suggest a 486 with

8 megs of RAM and a 100 MB hard disk as a start) and the fact

that you have to be quite familiar with Unix administration to

install the system.

DxNet is a DOS software with a PacketCluster-type user interface.

It requires the G8BPQ switch like Clusse does. It can be found

on the Internet at ham.ireste.fr, in the directory

/pub/Hamradio/DxNet/. The author, Olivier Le Cam, F5MZN, can be

contacted on the Internet as olecam@imaginet.fr or on packet as

F5MZN @ F6KPF.FBRE.FRA.EU.

I've also heard of another German project called DxNode. DxNet

was formerly called DxNode, but Olivier changed the name after

hearing of the other DxNode, which was there before Olivier

started to distribute his version...

��� 2. Installation �����������������������������������������������������

2.1 First installation

I think the program is meant to be installed by people already

familiar with BBS software and packet networking. Before installing

this program get your BPQ node or PC/FlexNet running, don't bother

me with requests for help concerning it... If you can't install it,

you won't be able to install this one... sorry. For an experienced

sysop it should be easy to get this up & running.

The latest G8BPQ switch code can be found on the Internet on

ftp.funet.fi:/pub/ham/packet/g8bpq/ or the Clusse web page,

http://zone.pspt.fi/clusse/. PC/FlexNet is not (i repeat NOT)

publicly available on the Internet or the packet network, but

it is free and your nearest PC/FlexNet sysop can tell you where

you can get a copy. I cannot supply it to you. If you're on the

web, see http://home.pages.de/~flexnet/ - it's the official

FlexNet site maintained by the authors.

Decompress the clu????.zip package using PKUNZIP 2.00 or newer

in the directory you want to install Clusse in. Remember to use

the -d command line switch of PKUNZIP to create the necessary

directories. Saves you from some trouble. I run Clusse from

c:\clu\, it should be OK for you... but Clusse doesn't care about

the directory name.

Edit all of the .ini files and the files in the text directory.

They should be self-explanatory, there are comments in each file.

Remember to read them ALL & edit as necessary!

If you want the user help texts in english, move text\english.hlp

to text\clusse.hlp. The default file is in Finnish. Other languages

are available separately on the Clusse distribution sites. If you

translate the help file to your own language, please let me know,

so i can include it at the Clusse distribution sites or tell others

where they can get it!

2.2 Upgrading from a previous version

Take a backup of your clusse directory structure. XCOPY

c:\clu\ c:\clu.old\ /S should do it. Then extract the

whatsnew.txt file out from the .ZIP file: pkunzip clu????.zip

doc\whatsnew.txt. Read all the new sections in the end if the

file, and take note on what files have changed their format.

Extract the rest of the package with the PKUNZIP -d

parameter over your previous installation. PKUNZIP doesn't

overwrite any files without asking you first. Of the configuration

files, extract only the ones which have changed.

Read and edit all of the changed/new configuration files. Here

you might need to check some previous settings from the backups!

2.3 Some notes on configuring the BPQ switch

You might want to increase the T3 and IDLETIME values to

decrease traffic, i use T3=1800 (Check if the user is still

reachable every 30 minutes) and IDLETIME=21600 (if no traffic

on a connection for 6 hours, disconnect).

ENABLE_LINKED should be A, to allow console connections to node

use the console callsign defined in clusse.ini.

No 'COMBIOS' ports are used. Just delete the TNCPORT blocks, if

you don't need them for some other software (you probably don't).

The UNPROTO address does not affect anything, Clusse sends the

UI packets as raw AX25 frames trough BPQ. Use the unproto.ini

file to configure digipeater paths etc.

If you're running Clusse as the only application interfacing to

BPQ, set "APPLICATIONS=CLU", and Application number in clusse.ini

to 1.

If you're running Clusse and FBB (or some other software) on the

same computer, use "APPLICATIONS=BBS,CLU" and application number 2

for Clusse. FBB has the BBSCALL, and users must connect Clusse via

the CLU command on the node. You must also tell clusse to use

BPQ host mode streams that are not used by FBB. When FBB starts up,

see what stream numbers it initializes, then use the rest for

Clusse. In FBB, the INITTNC files are used to specify which

streams and application number FBB uses.

2.4 Some notes on using the PC/FlexNet kernel

The interface to the PC/FlexNet kernel is a *new* feature,

not tested nor fully functional. Console connections to the radio

side (CTRL-F2) don't work. The choice of using the PC/FlexNet

interface is made in CLUSSE.INI. There's also some difference in

the connection script files. When i've finished this thing,

i'll supply some more information.

If you run PC/FlexNet with Windows 95, load it from autoexec.bat.

To get rid of the annoying Performance Warning "A new MS-DOS resident

program named 'FLEXNET' may decrease your system's performance" on

startup add flexnet.exe to \windows\ios.ini. Otherwice, Win95 uses

DOS compatibility mode for disk access, which IS slow. The next

version of PC/FlexNet is promised to support Windows a bit.

��� 3. Running Clusse ���������������������������������������������������

3.1 Command line parameters for Clusse.EXE

-h : Parameter help

-b : Force BIOS video I/O

-v : Force direct video I/O

-c : Create clusse.chr, a "clean" character translation table.

-d : Use a different root directory for Clusse files. For example

"-dc:\clu\". Default is the executable directory.

-e : Disables expiry

-m : Disables monitoring code completely

-n : Disables linking

-p : Wait for a keypress at startup before switching to windowed

mode.

-t : Use a different directory for temporary files. Like -d in

syntax.

-x : Disable XMS usage.

3.2 Using the console

3.2.1 Keys

���� Console keys: ���������� Ctrl- ������������� Alt-

F1   This help screen       �                   �

F2   Console connect        � Connect to node   �

F5   Toggle band monitoring � Action monitoring �

F8                          � Import DX spots   � Expire files now

F9   Start linking          � Disconnect links  � Hard disconnect links

F10  Shutdown               � Reboot software   � Dos shell

3.2.2 Console screen layout

There are two or three "windows" on the screen. The first window

is a so-called status window, which is 3 lines high. The second

window is the console receiving window, which shows the received

text, when the console is connected to the BPQ node (CTRL-F2) or

the local DX cluster node (F2). The monitoring window is only

available on EGA and VGA monitors in the 43- and 50-line modes

(for now).

Description of the status lines:

The first line tells the Clusse version number, the local/network

user and node/link counts and the date & time. The second line is

used for the progress meter (visible during expiry) and free/used

XMS values (if XMS is available).

Third one is the status line:

OH7LZB Logged in      BPQ 143 Heap fr 250648 us  47616 BA Ins

User ��     �                  �             �         �   ��  �

Action ������                  �             �         �   ��  �

BPQ free buffers count ���������             �         �   ��  �

Amount of free heap memory �������������������         �   ��  �

Amount of heap memory used �����������������������������   ��  �

Band, Action monitoring state �������������������������������  �

Insert/Overwrite keyboard mode ���������������������������������

3.2.3 Monitoring window

If Action monitoring is enabled (press CTRL-F5) to toggle,

the monitoring window shows a history of actions that are shown

on the third status line. If band monitoring is enabled (F5

toggles), the window shows all traffic on all BPQ ports.

Description of the band monitor header lines:

a) A simple AX.25 frame:

08:59:40 1:OH7RDA-2 � OH7AB-3  <I 17>

�   �     �        �       �  �

Timestamp �   �     �        �       �  �

BPQ port ������     �        �       �  �

Originating station �        �       �  �

Recipient ��������������������       �  �

Packet type ��������������������������  �

Sequence counters (tx, rx) ��������������

b) NET/ROM headers:

Information (I):

09:00:36 1:OH7RDA-2 � OH7AB-3  <I 11> NR OH7RDA-2 � OH7AB     I 4/174 1-1

L3 protocol ������������������������    �          �       �  �    � �

L3 originating node ���������������������          �       �  �    � �

L3 destination node ��������������������������������       �  �    � �

L4 frame type ����������������������������������������������  �    � �

L4 destination socket �����������������������������������������    � �

L4 tx sequence counter ��������������������������������������������� �

L4 rx sequence counter �����������������������������������������������

NET/ROM L4 frame types:

I  - Information

IA - Information Acknowledgement

C  - Connection request

CA - Connection Acknowledgement

D  - Disconnection request

DA - Disconnection Acknowledgement

3.3 Other general stuff

Always have your computer's clock in UTC time, if you're

connecting to a PacketCluster network. It uses UTC and i

don't do any conversions (yet). If your clock is not accurate

and is often more than just a few minutes behind or ahead, you

should consider either adjusting it (if possible) or installing

a TSR which can compensate the error (i use RIGHTIME). The system

time is used for timestamping DX etc. messages which are sent to

the cluster network, and wrong timestamps can confuse users and

the network.

3.4 Exit codes

Clusse leaves an errorlevel on exit. It can then be evaluated

by the batch file running Clusse (clu.bat), which can either

restart Clusse, run some external programs or exit. If the

errorlevel is < (smaller than) 10, clusse cannot run and should

not be restarted. If it is >= (larger or equal to) 10, Clusse

can *usually* continue operating and should be restarted

automatically.

0   - Normal exit. F10 pressed or SHUtdown command executed.

1   - Error in configuration file

2   - Invalid command line parameter

9   - Cannot run on this processor/OS version/memory manager/etc...

10  - File I/O error

11  - Out of heap memory

12  - XMS error

20  - BPQ or PC/FlexNet error

21  - BPQ running out of buffers

99  - Clusse internal run-time error

100 - Normal software reboot.

If you ever see number 99, report the problem to me. It's a bug

which needs to be fixed. ALWAYS remember to tell me the run-time

error number AND THE MEMORY ADDRESS. It's vital, only it can tell

me where to look for the problem!

3.5 Multitasking environments

Desqview: This program works under Desqview and uses the release

timeslice call if DV is detected. Direct video memory I/O is not

used under DV by default. You can also disable direct video with

the parameter -b, if using some other multitasker which doesn't

support fast direct video I/O transfers, and -v forces direct

video even if DV is detected. DV can handle direct video I/O when

used with QEMM.....

Windows 3.1: I tried it once, seems to work. Included is a sample

.PIF file, which should work OK. You may have to give it some more

base memory than originally specified in the PIF. Prevent Windows

from using the serial ports which are used by the BPQ switch.

Windows 95: Works just fine, Clusse is developed under it. BPQ

should be loaded in autoexec.bat, before the Windows multitasking

kernel. BPQCODE.386 is not required by Clusse, but Win95 needs a

keypress to start up, if it doesn't find the driver in the System

directory. Remove all of the serial ports you use for BPQ from the

Windows System Properties / Device Manager! The reboot cold/warm

commands cannot reboot the computer, as Win95 doesn't let a DOS box

do it. If you know a good way to get it done, let me know.

Windows NT: I have no experience over this OS, but what i've

written for Windows 3.1 and '95, should apply to NT... with the

exception that NT is stable 8-)

OS/2: I've heard it works, but did not try myself. Timeslice is

released when idle (trough the DPMI call).

Linux: No, i'm not working on a Linux version. I don't know

enough about Unix programming to start such a project. Perhaps

some day i'll find a good guy to do the porting job... OH1MMA

says that Clusse and G8BPQ worked under the DOS emulator of Linux.

He did not try using the serial ports yet. Anyway, you're wasting

resources if you run Clusse under the DOS emulator.... Linux is

TOO GOOD for Clusse! Try out clx! 8-)

3.6 Memory requirements

At the moment Clusse takes around 300 Kb of base RAM with no

users. Memory is allocated dynamically: the more users and nodes

in the network, the more memory you need. You can follow memory

usage by monitoring the status line and the STatus D, STatus M

commands. It doesn't show some peaks, like allocating a buffer

for a disk operation, then releasing it before the status line

is updated again. Data stored in XMS must be moved to base memory

before using it. 500 Kb of base memory should usually be enough.

At any time, clusse must be able to allocate up to 64 Kb of heap

for a temporary buffer!

On the status line you see three values that change during

execution. The first one is the BPQ free buffers count. The "fr"

value is the amount of available heap memory. When no heap memory

is available, Clusse exits with an error message and errorlevel 11.

The "us" value is the amount of heap memory currently allocated.

Clusse can use XMS memory for storing database index files. It's

up to you if you want to use your valuable memory by loading the

big index files to XMS and provide blazingly fast searches or save

the memory and do the searches on disk... the choise is made in

db.ini. The DXCC index file is loaded into heap if no XMS is

available and moved to XMS if possible (there's no way to control

that). Use the command line switch -X to disable all XMS usage.

��� 4. DX Cluster networking ��������������������������������������������

4.1 Linking to the PacketCluster network

This has probably been the hardest part of the project. The

protocol used is NOT documented too well, I've had to reverse-

engineer the system. But it seems to work now, rather well.

The neighbouring PacketCluster systems don't notice the difference

between a Clusse and a "real" PacketCluster system. Clusse does

not interfere an established PacketCluster network, so you can

link in without fear. Of course, your neighbouring node sysops

might be afraid... please be gentle!

Clusse can handle up to 10 simultaenous links to other nodes.

Clusse must initiate the links, it cannot handle incoming ones.

All messages are not forwarded (for example, the PC mail protocol

is not included in Clusse yet), but generally everything else

seems to work OK. See appendix B for more information on supported

messages.

An *IMPORTANT* note: Before linking to a PacketCluster, the

SysOp of the other end MUST tell his system that you should be

treated as a cluster node. This is done using the ADD/NODE

command. He should use the -BULLETIN parameter (eg.

ADD/NODE OH7RBA-1 -BULLETIN) to prevent PacketCluster from

trying to forward any bulletins to your system, as Clusse doesn't

implement the bulletin forwarding protocol.

4.2 Network topology

In general, the PC protocol was designed to be used in star-

and chain-formed configurations. It does not work in circle-shaped

networks. Let me demonstrate some possible network configurations.

Here's a simple chain:

OH7LZB-1���������OH7RBA-1���������OH1RBI��������OH2RBH

A star (a part of the Finnish network looks like this):

������OH6RDV

�

OH1RCF��ͻ       �����OH7LZB-1

�ͻ   �ͼ

�   �

OH1AI-6�����OH1RBI�����OH7RBA-1

�

�

OH2RBG�����OH2RBH���OH2RBQ

A circle would look like this:

OH6RDV������������������OH7RBA-1

�                       �

�                       �

OH1RBI������OH2RBH������OH2RBQ

DX Cluster networks usually consist of more complicated

combinations of stars and chains. The main problem in the

PacketCluster linking protocol is it's inability to handle

multiple routes to a node. Circle-shaped networks are not handled.

If you command the nodes to form a circle, messages start to

travel around it. Duplicates are not catched properly, and

the network gets overloaded. The situation is called a LOOP,

and must be avoided like a serious disease.

I've heard that the PacketCluster goes berserk very easily,

if it gets into a loop. Clusse does not. Duplicate DX spots

and WWV messages are filtered out. User and node addition

messages are not accepted, if the user or node is already in

the table. Deletions are not forwarded, if the user or node is

not in the table. And so on. But Clusse cannot make the existing

protocol any better, so loops are not allowed. If Clusse detects

it is in one, it disconnects the link it comes from, just to

protect the rest of the network.

4.3 Listen-only links

This makes an exception to what i just said about loops. Clusse

can create link in a listen-only mode, in which it only receives

DX spots, announcements and WWV information from the link, and

sends nothing back. The received information is not sent to other

links. Duplicate data is not taken in at all. This makes it

possible to have multiple links to the network, without creating

a cluster loop.

Here's a real-world example (OH7RBA-1 is my local node):

�����OH8RCA

�

OH6RDV      OH7RBA-1

�                    OH4RBB

�                      �

�           OH2RBQ   OH5RBX

�               �      �

����OH1RBI����OH2RBH��ͼ

OH8RCA is located in northern Finland. OH6RDV on the west coast.

OH1RBI is in  the southwest corner. OH2RBH and OH2RBQ are at the

south coast. OH7RBA is in mid-eastern Finland, OH4RBB and OH5RBX

are between OH7RBA and the west coast. Trough the Internet

gateways at OH7, OH1, OH2 and OH6, i can get a fast link either

to OH1RBI, OH2RBQ or OH6RDV. Using the VHF/UHF network (1k2 and

9k6 bit/s links) i can get to OH4RBB and OH8RCA. All the other

nodes are linked via the VHF/UHF network.

Due to the lousy condition of the network, either the link from

OH1RBI to OH2RBH or the one from OH1 to OH6 is broken most of the

time. If it works, the round-trip time from OH2 to OH8 is probably

around 20 minutes. I'd like to get the DX spot in before the station

moves off from the frequency. Where would I link OH7RBA-1, if i

could link to one place only??? (Because Clusse doesn't forward

PacketCluster mail, and the PacketCluster users still think they

need it, we cannot configure OH7RBA-1 as the central node for a

star-shaped network.)

.... got the point ?  . . .  Good.

I create a "normal" link to OH1RBI, and listen-only links to

OH2, OH4 and OH6. This way, i get most of the spots in reasonable

time, and the network is happy. The only problem is that all

messages originated from OH7RBA-1 and messages that are pointed

to OH7RBA-1 only (like "talk" messages) must travel via OH1RBI

only. Using the Talk feature for chatting to the other end of the

network is not fun at all.

When initiating a listen-only link, Clusse must announce it's

node callsign, otherwice the other end won't send any messages.

This callsign cannot be the one that is used for the normal link

or any other listen-mode links, because the PacketCluster would

then see two routes to the same node. The second callsign must be

specified on the I line in links.ini.

... Yeah, listen-only links were not my idea. I stole it from

the clx authors. Thanks, guys, and keep up the good work! Feel

free to steal some of mine, too! 8-)

4.4 PacketCluster version compatibility

Clusse has been succesfully linked (at least) with PacketCluster

versions 5.3-05, 5.4-32 and 5.4-77. All versions after 5 should be

OK. The PC linking protocol changed significantly when version 5

came out, so you WILL have trouble if you try linking with a v4

system.

I've successfully linked Clusse to a clx system. Linking with

DXNet running on the same computer didn't work on first try.

4.5 Linking two Clusse systems

Sorry. This is NOT possible. Yet! Of course, I'm planning to get

this running as soon as possible! I'm NOT going to support PC

protocol links between 2 Clusses. Just because the PC protocol is

so dumb and poorly documented and eats up far too much bandwidth.

I've been thinking about my own, binary (not human-readable) and

possibly packed (if i get some good code to compress small,

100-5000 byte packets with good compression rates). Yeah, it will

allow circle-shaped networks too.

��� 5. Files ������������������������������������������������������������

5.1 File descriptions and paths

o  Document files:  clu\doc\

intro.txt    - A quick introduction, "fact sheet" for Clusse. When

someone asks you a dumb question like "what the heck

is Clusse", mail this one to him.

clusse.doc   - This file. A kind of manual for the program.

whatsnew.txt - Revision history. See the end of the file for additions

in the latest versions when you're upgrading from a

previous version.

clusse.faq   - Frequently Asked Questions (and the answers).

db.ini       - A sample database configuration file with examples for

all of the supported database types.

o  Configuration files: clu\

clusse.ini   - This is the main configuration file. Vital!

links.ini    - Here you configure the packetcluster links. (1 (2

Delete this to disable linking.

expiry.ini   - Controls how long the records are stored in different

data files. (2

unproto.ini  - Configuration for the multicasting user interface. (2

db.ini       - Database configuration. This file is in the doc\ directory

by default, you have to move it here if you want to use

databases. (2

charsets.ini - Character conversion table config. You may delete this

if it's unnecessary. (2

password.ini - Password for remote sysop access.

o  Text files for users: clu\text\

clusse.hlp   - The help file for users (1

This file is in Finnish, an English version is included

as english.hlp - rename it to clusse.hlp if you want

(for some reason, most of you do).

fortunes.txt - The fortune database file (1 (2

info.txt     - Info file. Users can get this using the I command.

Tells something about the station, equipment, the

sysop etc... (2

login.txt    - This is sent to each user on login, right after the

QTH/information string in clusse.ini. Length limited to

1700 bytes. (2

news.txt     - System news file. Use this to notify the users of changes

in the software, club news etc... See the file for details

on syntax. (1 (2

newuser.txt  - This file is sent to each user when he connects the local

node for the first time (no record found in users.clu). (2

Notes:

(1: Don't edit these files while the program is running (in dos shell

a multitasking environment) - they have an index file which is

only updated at startup.

(2: You can delete this file if you don't need the feature, Clusse

won't mind.

o  Log files: clu\log\

clogyyyy.mm  - yyyy is the year, mm is the month. These are the logs.

error.log    - Run-time errors are logged here.

o  Binary files: clu\data\

announce.clu - Announcements are saved here...

convpers.clu - Convers /personal texts

dx.clu       - The DX file

frtnindx.clu - Index file for fortunes.txt.

helpindx.clu - Index file for clusse.hlp. Reindexed if the date of the

help file has changed.

last.clu     - Stores a list of last users and happenings (crashes, etc).

mid.clu      - Message ID table

netusers.clu - Network user data

nodes.clu    - Network node data

status.clu   - Counters and position pointers for other files (mid.clu).

users.clu    - Local user data

www.clu      - WWV announcements

You can delete any binary file you (don't) like. Clusse will

create a new one. Don't try to edit them if you don't really

know what you're doing. The data and log directories are created

by the program. The rest are not.

o  Database files: clu\db\

cty.db       - The CT-format DXCC database in ASCII

dxcc.cdb     - DXCC database in binary format

dxcc.idx     - An index file for the previous one

*.ful        - A PacketCluster-type database

*.idx        - An index file for a database

*.inf        - Information file for a database (in ASCII, required,

written by the sysop)

I recommend using a good disk caching program. One with a

write-back cache (Smartdrive NO GOOD!). I prefer HyperDisk.

NCache2 does a good job. A lot of Clusse actions include disk

operations, and with a slow system they can be painfully slow.

5.2 Databases

Databases are stored in the clu\db\ directory. They are ascii

files, which all can be edited by the sysop. When starting up,

Clusse first reads db.ini, then checks all ascii database files

and their respective index (.idx) files. If the database is new

(no index file found) or the date/time records of the files differ,

Clusse rebuilds the index.

Databases are configured in db.ini. The sample configuration

file is in the doc\ directory. Move it to the Clusse root directory

and edit it to suit your needs.

Each database also needs an information file to tell users, how

the particular database is used and what does it include (see

db.ini). This .inf file is written by the sysop. It's a plain

ASCII file. It's shown to the user when he issues a Find command

for the database, but doesn't specify a keyword to be searched for.

You, the sysop, must usually create the file.

The DXCC database, included in the distribution, is an

exception. It is not necessary to configure it in db.ini! It is

also converted to binary format (dxcc.cdb) to make reading at

run-time easier. It is used trough the Prefix command, not the

Find command!

5.2.1 PacketCluster-type databases

The database system of Clusse is compatible with the PacketCluster

databases, for example the DB0SDX QSL + address database. As a

PacketCluster imitation, it is extremely simple and stupid. Only

one (!) keyword per information block is allowed (i'll fix this one

soon). A line "&&" marks the end of a block. The first line of a

block includes the keyword, and the subsequent lines include the

information related to the keyword. Keywords are not case sensitive.

Here's two sample blocks in a database file:

---- clip clip ----

keyword

Information related to the keyword

more information related to the keyword

even more information related to the keyword

&&

anotherkeyword

Information related to anotherkeyword

more information related to anotherkeyword

even more information related to anotherkeyword

&&

---- clip clip ----

5.2.2 The DB0SDX QSL database

The sample db.ini file included in the distribution is the

configuration for the DB0SDX QSL and address database. The

database includes over 25 000 QSL information entries and over

10 000 mailing addresses.

The database is available on the internet via anonymous ftp

on this site:

ifsws1.sozialforschung.uni-stuttgart.de:

/pub/packclus/db/dbyymmdd.zip

The database is maintained by Lothar, DL1SBF, who can be

contacted using packet radio at the address DL1SBF @

DB0SDX.#BW.DEU.EU. He doesn't have an internet address, but you

could try to reach him trough an internet-packet gateway using

the internet address dl1sbf%db0sdx@db0fho.et-inf.fho-emden.de.

It's a one-way route, so don't except replies via the Internet.

Frank, DL1SBR takes care of distributing the database, and

can be mailed at ross@ifsws1.sozialforschung.uni-stuttgart.de

or DL1SBR @ DB0SDX.#BW.DEU.EU.

5.2.3 The DXCC database

Clusse supports the DXCC database format of the K1EA CT logging

software. The latest CTY.DAT file is always distributed with

Clusse. It is maintained by Jim Reisert, AD1DC. THANKS for

allowing me to use the file in Clusse, Jim!

The latest CTY.DAT can always be found on the internet via

anonymous FTP from the following sites:

ftp://maspar.maspar.com/pub/k2mm/ct-files/cty-??????.zip

ftp://ve7tcp.ampr.org/software/ct/cty-??????.zip

where the ?????? is replaced with the date of the distribution.

I try to include the latest version in the Clusse distributions.

If you have updates for the file, please mail them to Jim,

reisert@eng.pko.dec.com. Do NOT mail them to me!

Description of the CTY.DAT file format, exctracted from

HISTORY.CTY of the original database distribution:

--------------------------------------------------------------------

CTY.DAT is a new file format for CT Version 9.  It includes more

information than the .CTY files for previous versions of CT.  This

allows CT to calculate, in real time, beam heading and sun times.

It also means that a single file can be used for all DX contests.

The format is as follows.  Note that the fields are aligned in

columns and spaced out for readability only.  It is the ":" at the

end of each field that acts as a delimiter for that field:

Column  Length  Description

------	------	-----------

1	  26	Country Name

27	   5	CQ Zone

32	   5	ITU Zone

37	   5	2-letter continent abbreviation

42	   9	Latitude in degrees, + for North

51	  10	Longitude in degrees, + for West

61	   9	Local time offset from GMT

69	   6	Primary DXCC Prefix (A "*" preceeding

this prefix indicates that the country

is on the DARC WAEDC list, and counts

in CQ-sponsored contests, but not ARRL-

sponsored contests).

Alias DXCC prefixes (including the primary one) follow on consecutive

lines, separated by ",".  If there is more than one line, subsequent

lines begin with the "&" continuation character.  A ";" terminates

the last prefix in the list.

The following special characters can be applied to an alias prefix:

(#)	Override CQ Zone

[#]	Override ITU Zone

<#/#>	Override latitude/longitude

{aa}	Override Continent

--------------------------------------------------------------------

5.2.4 The Buckmaster HamCall CD-ROM callbook

Clusse supports the HamCall CD-ROM callbook published by Buckmaster

Publishing. The CD-ROM must probably be from April 1995 or later

distribution. Clusse requires that BUCKTSR.EXE is loaded. It is

located in the \API\ directory of the CD. Older CD's don't include

this utility.

See db.ini for details on configuration... Be sure to use the

LATEST version of BUCKTSR.EXE you have! When you purchase a new CD,

use the version on it.

The CD-ROM can be obtained from Buckmaster Publishing, Route 4,

Box 1630, Mineral, Virginia 23117. Their email address is

info@buck.com. They also have a web page at http://www.buck.com/.

I'd like to thank Buckmaster Publishing for donating me a copy

of the CD (for development purposes).

5.2.5 Other CD-ROM callbooks

Mail me an accurate description of the databate format, the

preferred ways to search a record from the database, and a copy

of the CD (i need it for testing), and i'll support the CD.

5.2.6 Other databases available (the ones i know of)

Chuck Strobel, K6PBT maintains a set of more or less interesting

PacketCluster-type databases, including an IOTA table and a Contest

calendar. They're available on the Internet at ftp.pinsight.com,

directory /pub/K6PBT/.

��� 6. Multicasting user interface ��������������������������������������

I've been working on an FBB-stylish unproto interface for

the DX cluster system. At the moment, DX spots, announcements

and WWV data are sent as UI packets. Unproto list requests and

uplinking are being planned. Unproto packets are sent as raw

frames trough the BPQ switch.

The ports and digipeater paths used for sending UI packets are

configured in unproto.ini. You can rename or delete this file to

completely disable the feature.

Messages originated from the node to all listeners are sent to

address 'DX'. Answers for requests have the recipient's callsign

as the recipient address. End-user software should listen to all

UI frames originated from the node, and make use of the node's

responses addressed to other users.

The protocol is completely my own. No other software uses it

yet, so it still can be modified without any problems.

Suggestions from other software developers are welcome.

Especially, if someone is interested in supporting the protocol

in a terminal/logging program...

The protocol has intentionally been designed to allow people

utilize it using a dumb TNC and a terminal. Just set it to monitor

all UI packets from the preferred node callsign to 'DX' and make

your TNC send UI packets addressed to the node.

UI broadcast packet format:

DX:       'D Index Freq DxCall TimeZ Date FromCall Info'

Ann:      'A Index FromCall Distr TimeZ Date Message'

WWV:      'W Index Date Hour SFI A K FromCall Forecast'

Description for the fields:

Index:    The index number of the message. Used for requesting lists.

Each message type has it's own index numbering. Wraps around

after 9999, to 1.

Freq:     The frequency, in KHz.

DxCall:   The DX callsign. (!)

TimeZ:    Time in UTC. Without a Z. Just "0140".

FromCall: Who submitted this message.

Info:     The additional info field for DX spots.

Date:     Date, MMDD, "0321".

Distr:    Announcement distribution area, * = full

Preliminary formats for messages from users to the node,

packets addressed to the node callsign:

DX:       'D Freq DxCall Info'

Ann:      'A Distr Message'

WWV:      'W SFI A K Forecast'

Dx req:   'D? Count' for last (count) spots.

'D? #Index' for one spot.

'D? #Index-Index' for several spots.

'D? #Index-Index #Index #Index #Index-Index' combinations OK...

Ann req:  'A? ' ... the way LD works ...

WWV req:  'W? ' ... the same for WWV ...

If the request from the user went "out of range" somehow (eg.

user specified an index number larger than the index of the

latest message, or requested too many old messages), the node will

answer with a packet 'DI FirstIndex-LastIndex' (or AI or WI)

specifying the available index numbers, and then send the messages

it can. The index packets can be requested with commands 'DI?',

'AI?' and 'WI?'.

To find DX cluster nodes supporting the UI interface, the user

can send a packet with the word 'QRZ' in the data field, addressed

to 'DX'. Nodes respond with a packet addressed to 'DX':

'I FirstDI-LastDI FirstAI-LastAI FirstWI-LastWI WW-Locator Software-ID'

specifying the index numbers for the latest messages. The WW

locator can be used for pointing antennas etc... The software ID

could be a (max) 12-character string defining the node software

brand and version. I'll reserve 'Clusse-030a' for Clusse 0.30a...

This Information packet can be requested using the command 'I?',

and is also sent when the node starts up.

When a node goes down (crashes, shuts down, reboots), it should

send a packet 'Q' to inform clients of the downtime...

Remember, the index numbers wrap from 9999 to 1, so the first

available index number may be larger than the last one... If it's

0, there are no spots available.

��� 7. Troubleshooting ��������������������������������������������������

Errare humanum est.

Run-time errors: You might get one of these, due to the bugs in

the program. Remember, all computer programs are written by human

beings, who tend to make errors. This is really an early version

(far from 1.0), so crashes can be expected from time to time 8-)

If you get a run-time error, tell me the error number AND the

address, and the situation when you got the error. Always remember

to mention the version of Clusse you're running, and before

reporting, see the whatsnew file of the latest clusse version

and check if i already have fixed the problem! Most run-time errors

are logged to error.log, which can be revieved using the LErrors

command. I appreciate error reports very much, i can't fix an error

i don't know about!

��� 8. Getting the latest version & contacting the author ���������������

8.1 Clusse distribution sites

The main distribution media for Clusse is the Internet. The

latest version of Clusse is always available on these sites:

WWW: http://zone.pspt.fi/clusse/

http://zone.oh7rba.ampr.org/clusse/ (ONLY IF you really are

behind amprnet routing!)

FTP: ftp.funet.fi: /pub/ham/packet/bbs/cluster/clusse/clu????.zip

zone.oh7rba.ampr.org: /pub/oh7lzb/clu/clu????.zip (Only if...)

???? is the version number. 030 means 0.30, NOT 3.0 ! Yes, it's

still FAR from 1.0 !  8-)

If you don't have Internet access, and you don't know anyone

who has, you can send a formatted 1.44" diskette and a SASE to

my address below. SASE is a Self-Addressed, Stamped Envelope.

Include the postage in Finnish or US funds! Please no IRC's.

Mailing a diskette overseas from Finland can cost up to 4 US

dollars (depends on the package), so a single greenstamp might

not be enough to cover my expenses.

8.2 Clusse mailing lists

If you're on Internet, and you have a valid email address, you

can subscribe to the Clusse Announcements mailing list by sending

a message to clusse-ann-request@iwn.fi with the word "subscribe"

in the title of the message. If you want to suscribe some other

address than the one you're sending the message from, just add

the email address, eg. "subscribe hessu@pspt.fi".

The mailing list is used for announcing new versions of Clusse

and related utilities. It is moderated (by me), and is not

intended for public discussion. Any mail sent to the list address

(clusse-ann@iwn.fi) ends up in my mailbox, and i will forward

it to the list, if i see it necessary. The list is a good & easy

way to keep up with new releases. If you have written or found an

utility program, which might be of interest for system operators

running Clusse, you can announce it on this list.

To unsubscribe, send a message to clusse-ann-request@iwn.fi,

with the word "unsubscribe" in the title. Simple.

8.3 The author (that's, me!)

If you have a question, please read the file clusse.faq, and

see if it is already answered there. People tend to ask the

same questions over and over, and the Frequently Asked Questions

(FAQ) file is meant to reduce unnecessary message traffic. If your

question is not answered in this document or the FAQ, or you have

some kind of feedback (positive or negative), feel free to mail

me.

If you want to contact me for some reason, here are the

suggested methods:

Internet email (preferred): hessu@pspt.fi

Amprnet email (a good one): oh7lzb@gw.oh7rba.ampr.org

Ax.25 packet (sloooow....): oh7lzb@oh7rba.#kuo.fin.eu

World Wide Web (graphic!!): http://zone.pspt.fi/~oh7lzb/

Snail mail:

(Always include a SASE if you want me to answer your letter!

I'm a poor student 8-) Please no IRC's.)

Heikki Hannikainen

Blueberry Road 17

70280 Kuopijo

Finland

Don't hesitate from sending money, diskettes (1.44" HD

preferred), beer, pizza, email, QSL's, whatever you think might

please me and make me fix another bug... A funny postcard can

save one's day!

��� Appendices: A - Things To Do ����������������������������������������

Please note that this list is NOT in order of priority! If

something is already mentioned here, there's no need to ask me

to work on it...

- Kill all the bugs. At least all I can find. When you see one,

report it!

- Load the Ann & WWV listings to memory (XMS, if available) to

speed up searches.

- Finish the multicasting user interface protocol.

- DX & ann & talk filtering system for users (need lists).

- An additional transmit buffer queue to prevent BPQ from running

out of frame buffers.

- A new, better cluster linking protocol.

- A mail system (PBBS compatible?).

- Interface for user-written server programs (like the PG feature in

FBB).

- Support for multiple separate language & help files.

- Timed & remote execution of DOS commands and programs.

- Direction, distance, MUF, sunrise/sunset time calculations based

on the prefix database. I need some formulas for this!

- Speech interface: Clusse will actually TALK the DX spots trough a

simple LPT port DAC to an FM transmitter.

- More remote sysop commands. More, more, more!

- Backscrolling for console windows, separate monitoring of a

particular user / link.

- Automatic reconnection of users disconnected due to a Clusse crash,

reboot or shutdown. Will only work for users connected directly (one

L2 hop) to Clusse.

Bugs I am aware of:

- On CGA and MDA monitors, the screen is sometimes garbled when

returning from DOS shell.

��� Appendices: B - PC protocol message description ���������������������

This information is based on the original PacketCluster documents

and my own observations (the documents are not too accurate -

several fields are missing). It's only valid for PC versions 5.?,

the version 4 protocol does not have all the fields. Clusse does

not support all of these message types.

As you can see, the protocol is not defined well enough. The basic

idea is to broadcast all messages to all links you have, except the

one it came from, or you can route it somewhere (talk messages,

pings, single-node announcements, mail etc).

Clusse does not even try to interpret and take action on all

of these messages... at least, not yet. I've marked the ones

actually supported with a '*' and the ones which are forwarded from

link to another but not looked into with a '+'.

One of the weird things is that if you don't have a value for

a field, you cannot leave it empty, every field MUST include at

least one space (20h).

Another funny thing is that you can send a alphanumerical STRING

instead of a numerical value. PacketCluster allows any string as

a frequency, but i don't suggest you to play with it, you may

crash the PacketCluster network!

*** BUG! ***

There's a real BUG in handling hop counts on node add messages.

When PC19 is used to send several node records in one message

(when links are initiated), the message has only one hop counter

(99 in the beginning). The nodes are farther away than that, and

the entries are distributed too far away, and often they are not

cleared (PC21 may not be distributed that far) when links fail.

When a new link is initiated, all PC16 messages are sent farther

than they should be (they get a hop count of 99). And the entries

are not cleared when these users leave the system.

Clusse comes around the second problem by storing the "real"

hop count for each node (based on PC50). When Clusse initiates

a link to a neighbouring node, it uses that hop count when

sending PC16's to the new link. It is also used for PC16's

(which are split up to pieces with the correct hop count for each

node).

***

PacketCluster usually forwards a protocol message before trying

to interpret it, if it knows where to put it. If a message is

corrupted somehow (RS-232 lines dropping characters, band noise,

AX.25 does not notice all errors), the corrupted message is

distributed trough the network. If you write an implementation

of the protocol, PLEASE try to catch corrupted messages before

forwarding them! Clusse converts all numerical values in messages

to binary format and catches corrupted ones, and also checks for

validity of all callsign fields.

Function                         Protocol Message

* Talk                PC10^FromUser^ToUser1^text^BellFlag^ToUser2^FromPc^~

* DX info             PC11^DXFreq^DXCall^Date^Time^DXInfoText^Logger^Hops^~

* Announcement        PC12^FromUser^ToPC^text^SysopFlag^FromPC^WxFlag^Hops^~

Stn into CONF       PC13^User^Hops^

Stn out of CONF     PC14^User^Hops^

Conference Mode     PC15^FromUser^text^hops^~

* Add user            PC16^FromPc^User ConfMode Here^User ConfMode Here^

User ConfMode Here^...^...^Hops^

* Delete user         PC17^User^FromPc^Hops^

PC link initialization & node table maintenance:

*   Request init      PC18^Cluster information (ignored!)^PCVersion^~

*   Add node          PC19^Here^PcCall^ConfMode^PCVersion^..more..^Hops^

*   Init done         PC20^

*   Delete node       PC21^PCCall^Reason^Hops^

*   PCDone            PC22^

* WWV info            PC23^Date^Hour^SFI^A^K^Forecast^Logger^FromPC^Hops^~

* Here status info    PC24^User^Here^Hops^

DX/WWV merge req.   PC25^ToPC^FromPC^DXCount^WWVCount^

* DX merge info       PC26^DXFreq^DXCall^Date^Time^DXInfoText^Logger^ToPC^~

* WWV merge info      PC27^Date^Hour^SFI^A^K^Forecast^Logger^ToPC^~

PC Mail:

SendSubject       PC28^ToPC^ViaPC^to-stn^fromstn^date^time^private-flg^

subject^?^?^?^?^OriginatedFromPC^~

SendText          PC29^ToPC^FromPC^msg-#^text^~

AckSubject        PC30^ToPC^FromPC^msg-#^

AckText (must be sent every 5 lines)

PC31^ToPC^FromPC^msg-#^

CompleteText      PC32^ToPC^FromPC^msg-#^

AckCompleteText   PC33^ToPC^FromPC^msg-#^

Remote commands:

Command           PC34^ToPC^FromPC^Command^~

Response          PC35^ToPC^FromPC^Command response^~

Show command    PC36^ToPC^FromPC^Command^~

Needs database upd  PC37^User^Command^~

* Connected node list PC38^node,node,node,...^~

* Delete node & disc  PC39^PCCall^reason^

PC file forward     PC40^ToPC^FromPC^filename^bulletin-flg^

* Addit. user info    PC41^User^InfoType^info^hops^~

Abort forwarding    PC42^ToPC^FromPC^msg-#^

PC Mail - external mail

SendSubject       PC43^ToPC^FromPC^date^time^private-flag^subject^~

* Remote DB request   PC44^ToPC^FromPC^stream^qualifier^key^fromstn^

* Remote DB response  PC45^ToPC^FromPC^stream^info^~

* Remote DB complete  PC46^ToPC^FromPC^stream^

* Remote DB update    PC47^ToPC^FromPC^user^qualifier^key^

+ Remote user DB upd  PC48^ToPC^FromPC^stream^qualifier^key^user^

Bulletin mail del   PC49^stn^subject^hops^~

* Update user count   PC50^FromPC^UserCount^Hops^

* Ping                PC51^ToPC^FromPC^PingFlag^

Description

Logger        Station reporting the info

PC            PacketCluster node callsign

User          The user's callsign

DXcall        DX station call

DXfreq        DX station frequency

DXmisc        DX misc text

stn           Call of connected station

from-stn      originating station

ConfMode      * = conf,  - = no conf

here          1 = here, 0 = nohere

bell-flag     1 = bell, 0 = no bell

PingFlag      1 = This is a ping, 0 = this is an answer for a ping

WxFlag        1 = this is a wx message, 0 = a normal announcement

Time          is in format hhmmZ where hh is hours, mm is minutes,

including the leading zeroes.

InfoType      1 = Name (22 characters), 2 = QTH (long enough),

3 = Coordinates, 4 = Home node callsign

?             I have no idea what these fields are used for. If

you have, please tell me!

��� Appendices: C - Clusse binary files description ���������������������

Here are the record types for the binary files used by Clusse,

in Pascal. All strings are pascal-type (the first byte indicates

the actual length of the string). Times are packed datetime

records as used in the DOS FAT file system (32 bits per record,

accurate to 2 seconds). LongInt is a 32-bit signed integer, Word

is a 16-bit unsigned and byte an 8-bit unsigned integer.

CallRec    = String[10];

LUserFRec  = Record { users.clu }

Call,                   { Callsign }

Name      : CallRec;    { Name }

Locator   : String[6];  { WW locator }

Personal  : String[40]; { Some personal description }

Logins,                 { How many connects }

Time,                   { Last logged in/out (packed datetime) }

NewsTime  : LongInt;    { Unused. }

Messages,               { Messages flags }

Beeps,                  { Beeps flags }

Flags,                  { General flags }

Rights    : Word;       { Permission flags }

CharSet,                { Character conversion table }

Prompt    : Byte;       { Prompt type }

End;

NUserFRec  = Record { nusers.clu }

Call      : CallRec;    { Callsign }

PC        : CallRec;    { Home node }

Name      : CallRec;    { Name }

QTH       : String[40]; { QTH }

PCQuality : Byte;       { How did we get the home node info:

0 - No idea (unreliable)

2 - DX/ANN/WWV etc message

5 - User login message

7 - Set manually by user or sysop

Information is replaced, if the Q

is higher than in the old record. }

Time      : LongInt;    { Last changed }

Index     : LongInt;    { Position in the file - ONLY used

internally by Clusse when reading

and writing the file, to speed up

actions. Not used when the record

is stored on disk. }

End;

HostFRec   = Record { nodes.clu }

Call    : CallRec;   { Callsign }

Via     : CallRec;   { Heard via }

Hops    : Byte;      { How many hops }

HopsOK  : Boolean;   { Is the hops count valid }

Ver     : Word;      { Software version }

Rtt     : LongInt;   { Round-trip time (sec), -1 : unknown }

Time    : LongInt;   { Last changed }

End;

DxInfoRec = Record { dx.clu }

Num      : LongInt;      { Index number }

Time     : LongInt;      { Packed datetime }

Call     : String[15];   { DX callsign }

Freq     : Real;         { Frequency }

FromCall,                { Who submitted the spot }

FromPC   : CallRec;      { and on what node }

FromRoute: Byte;         { What link port did it come from }

Info     : String[80];   { Misc text }

End;

AnnRec    = Record { announce.clu }

Num      : LongInt;      { Index number }

Time     : LongInt;      { Packed datetime }

FromCall,                { From callsign }

FromPc,                  { From node }

ToPC     : CallRec;      { Distributed to node (* = full) }

FromRoute: Byte;         { What link port did it come from }

Critical,                { Is it a critical one }

Wx       : Boolean;      { Is it a WX spot }

Msg      : String;       { The message itself }

End;

WWVRec    = Record { wwv.clu }

Num      : LongInt;      { Index number }

Time     : LongInt;      { When we got it in }

Hour     : Byte;         { Hour of the WWV message }

SFI, A   : Word;

K        : Byte;

Forecast : String[80];

FromCall,                { Who submitted this one }

FromPc   : CallRec;      { ...on what node... }

FromRoute: Byte;         { What link port did it come from }

End;

HelpIndexRec = Record { helpindx.clu }

KeyWord : KeyWordType; { The help keyword }

Position : LongInt;    { Where is the actual help text }

End;

LastFileRec  = Record { last.clu }

Time      : LongInt;   { When }

Call      : CallRec;   { Who }

Event     : String[40] { What }

End;

StatusRec    = Record { status.clu }

Time     : LongInt;   { Last updated }

{ Position pointer for the MID file mid.clu: }

{ the actual position is MidPosT * 1000 + MidPos }

MidPosT  : Byte;

MidPos   : Word;

End;

mid.clu is a file of MidType = String[10];.

��� Appendices: D - Keeping the lawyers happy ���������������������������

- MS-DOS, Microsoft Windows and Windows 95 are registered trademarks of

Microsoft Corporation.

- DesqView and QEMM are registered trademarks of Quarterdeck Office

Systems.

- OS/2 is a registered trademark of International Business Machines, Inc.

- CT is a trademark of K1EA Software, a division of Harvard Radio.

- PacketCluster is a trademark of Pavillion Software, a division of

Harvard Radio.

- PKUNZIP is a trademark of PKWARE, Inc.

- HamCall is a trademark of Buckmaster Publishing.

- Any trademarks mentioned in this file but not included here by

accident are also acknowledged....