MSYS 1.02 Update

****** IMPORTANT *****

Beginning with this version, the forward hours line in the forward

file MUST begin with !

Old way			New way

F0 WA8BXN		F0 WA8BXN

10-22                   !10-22

WA8BXN                  WA8BXN

-----                   ------

You should replace your current programs with the ones on this disk.

LFA has been improved so that it should work with large amounts

of input data. Try to keep input file size to periods of not

more than 4000 messages having been passed!

A new program has been included that scans messages saved in mail.bak

directory format. It produces a list of bbs headers seen in the msgs.

It is called BBSLIST.EXE.

3-8-89

A problem that has been around for a while has finally been

identified. It deals with a bbs sending long messages to msys

using a path that is marginal with frack>1. The symptom is that

both stations go "wild" in flooding the channels with packets.

Here is what happens: The sending station sends out multiple

packets (call them 1 2 3 & 4). Due to noise msys does not receive

packet #1 but does get 2 3 & 4. It sends out a REJ 1 for each

of the packets received. For each of these 3 REJ packets the sending

station resends all 4 packets again. If MSYS misses packet 1

again each time (maybe its a long packet, or maybe the sending

station has a txdelay that is marginal) then it sends out the

REJ1 frames for all the other packets received (thats 9 REJ

frames this time). The whole thing quickly snowballs into a mess.

To hopefully avoid this situation, I have added the following

command to MSYS:

RESponsetime #   where # is the number of seconds to wait before

sending out a response. This is the T2 timer value in thE ax.25

spec. If you set RES to 0 it will work the old way, always sending

out a response to each I frame received as soon as possible.

With non-zero values for RES, the responses are held, the latest

one replacing any one being held, until no I frames have been

received for the time given. It then sends out its response. I'm

not sure this is going to work but am sending you this latest

version to let you help test the idea.

Two other commands have also been added, both dealing with

suspending the monitor function on the F2 screen (it takes a

very significant amount of time to keep formatting all those

packets displayed). The commands are:

F2 #       # is the number of seconds to continue displaying

monitored packets after F2 was last pressed. The default is 600

seconds (10 minutes). Thus 10 minutes after you last press F2

monitoring will be turned off until you press F2 again.

SUspendtrace #      Where # is the number of free bytes in the input

buffers below which the monitoring is turned off. There is 1 buffer

of 4096 bytes for each serial port (tnc). The amount free at any

time is given by the STatus command. When the free space gets

less than the SU value, F2 monitoring is turned off to allow

things to get processed more quickly. Default value is 2000 bytes.

Again, F2 displays take a lot of time and when the system gets

behind it is a good idea to turn off the updating of the display.

When the free space in all the buffers goes above the SU value

tracing is turned back on (unless otherwise suspended). This

function can be effectively disabled by setting SU to 0 and then

it will work as before.

3-17-89  Last minute changes

The BBS send command has been changed to allow to callsigns to

have 1 or 2 digits and still be treated as valid callsigns. It

used to be just a single digit, thus treating some foreign calls

of the form 5U4GB as a bulletin.

Changes to keyboard connections: When you are connected to a

station on your keyboard call (either you made the call or

answered one) if you type (at the beginning of a line)

\+\ filename

that file will be sent to the station you are connected to as if

you typed it. Example:

\+\ files\stuff

would send the file called stuff from the files subdirectory.

A change has also been made to the capture function. When you

turn capture on with the \\\ at the beginning of a line, you

can now also specify a filename to be used in place of the

default CAPTURE.DAT. For example you can type

\\\ junk.abc

to start capturing your conversation into the file JUNK.ABC.

When you have already turned capture on during a connection,

if you type something like

\\\morejunk.xyz

then the current capture file will be closed and now your

conversation will be saved in the file MOREJUNK.XYZ. As before

any open capture file is closed when the connection is terminated.

+----------------------------------------------------------------+

| Future updates of MSYS on disk directly from WA8BXN will be    |

| available to stations for $5 each. This includes the cost of   |

| postage, the disk mailer, and the disk itself (all provided    |

| for your $5). Its just getting too expensive for me to foot    |

| the bill for all the stations using MSYS myself. Please send   |

| money only: disks may get damaged in the mails and are not     |

| very expensive these days (the mailers cost a lot more!) If    |

| you insist on sending disks, then also send self addressed     |

| stamped mailers (US postage) for each disk.  You may also      |

| get copies from stations that have already gotten an update by |

| making arrangements with them. Of course the most up to date   |

| versions come directly from WA8BXN (Name and addr good in any  |

| call book). (Canadian stations send $7.50 to make up for the   |

| difference in postage & exchange rate)                         |

| Send requests to:      Michael Pechura                         |

|                        10809 Beechwood Drive                   |

|                        Chesterland, OH 44026                   |

+----------------------------------------------------------------+